Applications/possibilities

Large sized sight and sun protection in front of windows and façades in private and commercial living and business areas, especially for public buildings, administrations, commercial buildings, offices, hotels, schools, universities, meeting rooms and gyms.

Low noise movement of any kind of curtains due to angular magnetized rotor with minimum vibration/noise.

Decorative draperies or heavy black out curtains, also as a room partition or for smaller scenes.

Shapes/dimensions/stack positions

Straight or multi – bent rail

Special shapes on request

Bending radius minimum 350 mm

One part, two part or multi-part curtains

Stack position left, right or centre

Motor position left or right

Large curtains with tandem motor left and right

Optional: Inverse Mounting for “false ceilings” (motor up position)

Rail length, manual operation up to 7000 mm

Rail length, electric operation up to 21000 mm with connectors

Rails of more than 7000 mm with rail connector

Curtain height up to 5000 mm

Drapery weight up to 500 g/m2

Curtain weight up to 70 kgs (Tandem S-35)

Rail

Solid rail profile, one track, extruded as per EN 12020-2, length: 7000 mm

material EN AW-6060/AIMgSi0.5 F22. Surface: E6/EV1, naturally anodized or white powder coated RAL 9016

other coatings on request

Dimensions: width 32 mm, height: 26 mm

Guiding channels for conveyor belt

Special channel with rail aperture for Trietex runners and gliders

multi-bending without stud

for rails longer than 7000 mm: solid and torsion-free aluminium rail connector

Rail gear and counter bearing

Stable plastic box made of high tensile, durable polyacetale

ball-bearing driving axle and deflection roller

driving axle made  of special, torsion-free CuZn-alloy

drive wheel and deflection roller made of high tensile and resistant-to-wear plastic

screws and axles protected against corrosion

eyelets for fixing of drapery

colours: white or light grey

measurements box of rail gear: width 50 mm,

length 80 mm

measurements box of counter bearing: width 37 mm, length 57 mm

Conveyor belt

Trietex toothed drive belt with a special profile and steel inlay, black

especially resistant-to-wear due to twoside special coating

extremely low extension rate for an exact final positioning

safe and form-fit power transmission

low friction and high flexibility

very smooth, regular run, low noise

UV-proof

Curtain Track Components

Drive runner

Plastic glider and safety clip, white or light grey

rigid traverse joint, 100 % special steel, length: 182 mm

overlap up to 180 mm in case of two part curtain

Runner

Runner with turnable eyelet for curtain weights up to 300 g/m2

runner with rigid eyelet for curtain weights up to 500 g/m2

galvanized steel axle for a very smooth run with heavy weights

wheels with special surface, low noise

alternative: for straight curtain track rails up to 7000 mm: turnable Trietex glider

colours: white or light grey

10 runners or glider per running meter recommended

curtain track rail hooks, white

Mounting

Ceiling mounting with quick locking bolt made of galvanized sheet steel or bolting through the rail channel

wall mounting with adjustable, solid mounting angle made of sheet steel

Quick locking bolt mounted at moving part of angle.

Adjusting distance of angle: either 85-145 mm or 145 – 230 mm

Mounting angles in white or galvanized

Manual operation

Simple fixing onto rail gear by means of bayonet locking, locked

Fixing either left or right side of rail.

Ball chain operation,

length of chain 1500 mm or 2500 mm

or on request, ball diametre 4,5 mm

Colours: white or light grey as rail colours.

Electric operation

Monophase- alternating voltage synchronous motors 230 V/50 Hz with reduction gear

Running speed 130 mm/second or as a speed version 190 mm/second

Power: 35 Watt in combination with special angular magnetized rotor.

Alternative: 30 W, 45 W and 65 W single or as tandem drive with standard rotor.

End positions adjustable from outside, in case of tandem drive only at pilot motor

All shapes also available in speed version and in 110 V/60 Hz

Dimensions with rail gear width 60 x 50 mm x height 310/338/370 mm

Motors fixed vertically downward, covered by curtain, simple fixing onto rail gear with bayonet locking, locked, either on right or left side or in case of tandem drive at both sides of rail

Optional: Inverse fixation for false ceilings (motor position up).

Colours white or light grey as rail colours.

Accessories: motor coupling piece, fixing clamp for motor coupling, drapery fixing device

Protection: IP 40, only for dry rooms

Motor manufactured in Germany with 100% final control.

Certificates: EMV/CE (VDE for motors with standard rotor). DIN ISO EN 9001:2008.

Operation elements/control elements

Single operation with pushbutton/locking switch or radio remote control

Group control with:

Pushbutton/locking switch or radio remote control with group control unit and/or

overriding central control unit and/or brightness control

indirect application for BUS system control with optional actors.

Picture: